Congestion Insights API (Network as Code / Nokia) — Summary Notes

What it does

Unlike Geofencing/Location Retrieval/Location Verification (which all report on a device's position), Congestion Insights reports on the state of the network itself in the geographic area where a device is located. The core question it answers: "Is the network in this device's area experiencing (or about to experience) congestion?"

Underlying technology: NWDAF

- NWDAF = Network Data Analytics Function, a core 5G component that safely exposes core network data to Network Operators in real time.

- Network as Code acts as a mediator between the app and NWDAF, abstracting/simplifying access to this data for authorized data consumers.

Definition of "Congestion"

Localized network usage exceeding the network's ability to handle it without degraded performance (increased latency, reduced bandwidth).

Two access modes

- Webhook subscription — subscribe once, receive automatic push notifications when congestion level changes.

- Polling — call a method directly and get an immediate response with current or predicted congestion level.

Critical dependency: subscription is mandatory before polling

The docs explicitly state: subscribing is a mandatory prerequisite before polling or getting subscription info, because the subscription itself triggers congestion metrics collection for that device. There is no data to poll without a live subscription first.

Subscribing

congestion_subscription = client.congestion_insights.create_subscription(

device={"phone_number": "+999991234567"},

webhook={

"notification_url": "https://example.com/notifications",

"notification_auth_token": "my-secret-token",

},

subscription_expire_time=datetime.now(timezone.utc) + timedelta(days=1),

)

Parameters

| Parameter | Type | Description | Required? |
| --- | --- | --- | --- |
| device | object | Device to monitor congestion for | Mandatory |
| subscription_expire_time | datetime | When the subscription expires | Mandatory |
| webhook.notification_url | string | URL to receive notifications | Mandatory |
| webhook.notification_auth_token | string | Auth token for the notification URL | Optional |

Note: authentication here is nested inside a webhook object (notification_auth_token), a different structure/naming from Geofencing's separate sink_credential parameter (PLAIN/ACCESSTOKEN).

Subscription response fields

| Field | Type | Description |
| --- | --- | --- |
| subscription_id | string | Subscription identifier |
| startedAt / starts_at | datetime | When the subscription started |
| expiresAt / expires_at | datetime | When the subscription expires |

(Docs are inconsistent — sometimes shown as subscription_id/starts_at/expires_at on the returned object, sometimes as startedAt/expiresAt in a separate parameters table.)

Receiving webhook notifications

Example handler uses FastAPI. The auth token is sent as an HTTP header in the form Authorization: Bearer <token> — this is the one API among those covered so far where the docs explicitly show how the auth credential is transmitted.

@app.post("/notifications")

def receive_notification(notification: Notification, authorization: str = Header(None)):

if authorization == "Bearer my-secret-token":

print(notification)

Unsubscribing

client.congestion_insights.delete_subscription(resource_id="<resource_id>")

Note: the parameter is named resource_id here, though other examples refer to the same value as subscription_id/.resource_id on the subscription object — naming is not fully consistent across the docs.

Polling current congestion level

congestion = client.congestion_insights.query(

device={"phone_number": "+999991234567"},

)

first_congestion_object = congestion[0]

print(first_congestion_object.level)

- Returns an array of congestion level objects, not a single object.

- Possible congestion levels: None, Low, Medium, High.

Predictions / historical data

Same query() method, with optional start and end timestamps:

congestion_data = client.congestion_insights.query(

device={"phone_number": "+999991234567"},

start=datetime.now(timezone.utc),

end=datetime.now(timezone.utc) + timedelta(hours=3),

)

Parameters

| Parameter | Type | Description | Required? |
| --- | --- | --- | --- |
| start | object/string | Start of time window (ISO 8601 or datetime) | Optional |
| end | object/string | End of time window (ISO 8601 or datetime) | Optional |

- No start/end → returns current state only.

- start/end in the future → returns predictions.

- start/end in the past → returns historical data.

Query response fields (per time interval)

| Field | Type | Description |
| --- | --- | --- |
| time_interval_start | string | Start of the interval (ISO 8601) |
| time_interval_stop | string | End of the interval (ISO 8601) |
| congestion_level | string | None / Low / Medium / High |
| confidence_level | integer | Confidence (0–100) — relevant for predictions |

Getting/managing subscriptions

subscriptions = client.congestion_insights.list_subscriptions()

first_subscription = client.congestion_insights.list_subscriptions()[0]

one_subscription = client.congestion_insights.get_subscription(

congestion_subscription.resource_id

)

Getting subscription info also requires an existing subscription to have been created first (same mandatory-subscription rule as polling).

Webhook notification JSON schema

{

"id": "...",

"source": "org.camaraproject.congestioninsights.v0.event",

"type": "event",

"specversion": "1.0",

"datacontenttype": "application/json",

"time": "2025-06-30T08:54:25.240202Z",

"data": {

"timeIntervalStart": "...",

"timeIntervalStop": "...",

"congestionLevel": "Medium",

"confidenceLevel": 75

}

}

Note: fields inside data in the raw webhook JSON use camelCase (timeIntervalStart, congestionLevel), while the Python SDK's query() response objects use snake_case (time_interval_start, congestion_level) — a naming mismatch to account for when parsing both sources in the same codebase.

Comparison with Geofencing

|  | Geofencing | Congestion Insights |
| --- | --- | --- |
| What it monitors | A specific device entering/leaving an area | Network congestion state in a device's area |
| Tied to a specific device? | Yes | Yes (device is mandatory for both subscribing and querying) |
| Signals crowd density? | No (only binary presence of a known device) | Indirectly — congestion reflects heavy network usage, which can correlate with crowd density |
| Access mode | Push only | Both push (webhook) and pull (polling) |
| Future predictions? | No | Yes, via start/end in the future |

Practical implication for a crowd-safety use case

- Congestion Insights is still tied to one device per subscription/query — it reports the congestion experienced by that device's location, not an aggregate reading for an entire area independent of any device.

- To get a fuller picture of congestion across a zone, multiple "reference devices" distributed across the area would need to be subscribed/queried individually and their results combined — it does not natively provide an area-wide reading without anchoring to specific devices.

الـ device هنا هو الترمومتر — أداة وصول لقراءة حالة الشبكة في مكانه، مش إنه هو موضوع القياس نفسه.

REFERENCES:

- API Hub

- use case example in python